# Low-Precision Sensitivity in VLAs: Cross-Region Interaction and Flow-Step Effects in Flow Matching Action Heads

Ziyu Chen, Zheng Xu

zche0506@uni.sydney.edu.au, zhxu0272@uni.sydney.edu.au

# ABSTRACTION

Vision-language-action models (VLAs) have recently emerged as an important paradigm for embodied intelligence, but deploying them on resource-constrained robotics platforms remains difficult due to memory and inference-cost limitations. Post-training quantization (PTQ) is a promising approach because it can reduce model cost without requiring full retraining. However, VLA quantization is more complicated than standard LLM quantization, since small numerical perturbations can propagate into action errors and task failure.

Recent work such as QuantVLA [3] has shown that some DiT-based VLA action heads (such as in the model GR00T N1.5) is more sensitive after LLM’s quantization, they only managed to preserve the model by several calibration methods, which are not generally effective methods, shown in this study. Meanwhile, their studies typically quantized the action head mainly at the feed-forward block level, while attention-side action-head layers are often treated as more sensitive and left in higher precision. This leaves an open question as whether action head and their layers are inherently fragile or their sensitivity to quantization alike perturbation or low precision depends on the model architecture, quantization target regions, and more.

This study investigates Evo-1, a lightweight VLA with a flow-matching action head. Instead of treating quantization only as an efficiency method, we use it as a controlled perturbation probe to study where and when the flow-matching action process is sensitive. We examine two quantization target sets: a 114-layer QuantVLA-style base set, including the backbone’s LLM linear layers and action-head FFN linear layers, and a selected 16-layer set from the action-head attention module. We further studied timestep-window effects across the 32-step flow-matching process in this model.

We discovered that sensitivity is not purely about locations of linear layers. The selected 16 action-head attention linear layers are not fragile when quantized alone, but degradation can appear when they are combined with the broader 114-layer quantized base set. LLM activation quantization is much more destructive, while LLM linear layers weight-only quantization remains relatively safe. Timestep-window experiments show that earlier flow steps are generally more sensitive than later steps, although full-timestep quantization can remain strong in normal settings and may only fail more aggressively under stress-test conditions. These findings suggest that VLA quantization should be evaluated through region interaction, timestep behavior, and task-level outcomes rather than layer-wise similarity alone.

# INTRODUCTION

Recent VLA quantization studies have shown that low-bit inference is possible, but also that sensitivity is uneven across model components. QVLA [1] argued that small quantization errors can accumulate into large action failures in embodied systems. DyQ-VLA [2] showed that quantization sensitivity can vary over time during execution, motivating dynamic precision instead of one fixed quantization scheme. QuantVLA further demonstrated that PTQ can be applied to DiT-based VLAs such as OpenPI π0.5 and GR00T N1.5 by preserving more sensitive components and adding calibration to stabilize quantized inference.

However, these studies do not fully establish whether the same sensitivity patterns hold across different VLA architectures. In particular, QuantVLA-style settings on GR00TN1.5 focuses on quantize the LLM and the feed-forward blocks of the action head, while attention-side layers in the flow-matching action head are left unquantized. This does not necessarily prove that such layers are always too sensitive. It may instead mean that their sensitivity depends on the model architecture, model size, and benchmark various robot task settings.

Evo-1 is a useful case for studying this question because it is a lightweight VLA with a small LLM backbone and a flow-matching action head. During inference, the VLM context is computed once for an action chunk, and the action head then refines the action trajectory through a 32-step flow-matching process. This creates a different kind of low precision problem as the same action-head layers are reused across flow steps, so perturbation may have different effects depending on whether it is applied early, late, or across the full trajectory.

This study therefore investigates Evo-1’s quantization sensitivity from two axes. This study therefore investigates Evo-1’s quantization sensitivity from two axes: the region axis and the timestep axis. The region axis asks which quantization target sets can tolerate low precision and is it really about the targets themselves, while the timestep axis asks when low precision is more harmful inside the action-generation temporal trajectory. We tested whether the flow-matching timesteps are equally sensitive to low precision, or whether certain windows are more vulnerable. Our initial hypothesis was that later steps could be more sensitive, but the experimental results contradicted this guessing, although this kind of sensitivity may not be simply linear across the full 32 time steps.

# METHODOLOGY

## Quantization Regions

We divide the model into two quantization regions. The first region is a 114-layer base group, which includes the language/backbone linear layers and the action-head FFN linear layers. This region follows QuantVLA’s setting, where FFN-style linear layers are usually treated as safer quantization targets.

The second region is a separate 16-layer group from the attention-side linear layers inside the flow-matching action head. This 16-layer group is the focus of this study, which QunatVLA excluded for GR00T due to their sensitivity to quantization in their study. Unlike the language backbone, the action head is reused repeatedly during action generation, so quantization in this region can affect the action trajectory differently depending on the flow timestep.

## Flow-Matching Timestep Windows

During inference, the action head does not generate the final action in a single pass. Instead, it refines the action through a 32-step flow-matching process. At each step, the action-head layers predict an update direction for the current action state. Therefore, this work treats timestep as an additional quantization axis, not only layer identity.

To test timestep sensitivity, the 32 flow steps are divided into windowed settings. The main setting is four 8-step windows, but later extended to 16-step, 23-step, and entire 32-step larger windows, including quantization across all attention linear layers of flow matching action head inside the selected time steps. This design was designed to test whether low precision is equally safe across the whole flow trajectory or whether early, middle, and late steps have different sensitivity.

## Experiment Designs

Our initial experiments tested the two regions separately. For the 114-layer base group, we compare the effect of LLM/backbone weight and activation quantization. For the 16-layer action-head attention group, we compare no quantization, W8-only, A8-only, and W8A8 settings.

After the initial isolated experiments, we designed a combined-region setting. The 114-layer base group was quantized together with the selected 16 action-head attention linear layers. At the same time, we varied the timestep window in which quantization was applied to the 16 attention layers during the 32-step flow-matching process. This design combined the region axis with the timestep axis, allowing us to study whether the effect of quantizing the combined regions is also interacting with the temporal settings. In total, this main experiment evaluated nine modes across different selective quantization settings (W8F16, W16A8, W8A8) and timestep-window settings.

# RESULTS AND ANALYSIS

## Initial Experiments

The Major experiments were conducted using the LIBERO suites, including Libero_Spatial, Libero_Goal, Libero_Object, and LIBERO_10. Each suite contains 10 tasks, and we used 5 episodes for each task, for a total of 50 runs per suite.

The first experiment was conducted on the first region, which includes linear layers from the backbone LLM and the action-head FFN layers, for a total of 114 linear layers. This region shows a clear contrast between weight and activation quantization. A8W16 activation quantization was catastrophic. Even with QuantVLA’s calibration or correction on the DuQuant mechanism, it only improved cosine similarity from around 0.86 to 0.89. In contrast, W8A16 was very safe, with similarity around 0.9998 and accuracy close to, or slightly better than, the no-quant baseline.

As for the second region, the selected 16 linear layers from the action-head attention module were not sensitive compared to the no-quant baseline (Table I). In contrast to QuantVLA’s setting, where these layers were excluded due to sensitivity, Evo-1’s action-head attention linear layers showed no degradation by themselves. However, later experiments show that the combination of both regions may tell a different story and could explain why they were excluded in other studies. In Evo-1, the goal is not to determine whether the 16 layers succeed or fail by themselves, but to study potential accuracy degradation when they are combined with the 114-layer QuantVLA-style selection and quantized together.

TABLE I

Table 1. Effect of Target16 Quantization Type on LIBERO-Goal Performance

| Target16 | Accuracy | Mean RMS Dript |
| --- | --- | --- |
| FP16 Baseline | 0.94 | N/A |
| W8A16 | 0.953 | 0.002601 |
| A8A16 | 0.96 | 0.001582 |
| W8A8 | 0.96 | 0.002784 |

Meanwhile, with our various timestep-window setups, accuracy became as high as 96% when using the full timestep window, but there were not many clear findings regarding whether W8A8, W8A16, or W16A8 was used (Table I). This could be because this experiment was only conducted on the LIBERO-Goal suite, as the smoke test for the main experiment indicated that it was the only suite showing significant accuracy differences when the combined experiments quantized the entire 130 layers with W8A16 quantization.

Next, under the W8A16 quantized setting, the effect of flow-window coverage (Table II) shows a slight upward trend as the quantized window becomes larger. Although the mean RMS drift increases with wider flow coverage, this higher numerical drift does not directly translate into lower task success. Instead, the larger-window settings achieve equal or better success in LIBERO_Goal, suggesting that drift magnitude alone is not sufficient to explain behavioural degradation. In this case, broader quantization appears to be tolerated and may even stabilize execution compared with narrower timestep windows.

TABLE II

Table 2. Effect of Flow-Window Coverage on LIBERO-Goal Performance

| Target16 | Accuracy | Mean RMS Drift to FP |
| --- | --- | --- |
| FP16 Baseline | 0.94 | N/A |
| 24-31 | 0.94 | 0.001857 |
| 16-31 | 0.96 | 0.002110 |
| 0-31 (All) | 0.973 | 0.003000 |

## Combined Experiments

Because neither region showed strong degradation under the separate W8A16 setting when evaluated alone, we used the combined 130-layer W8A16 quantization setting as a baseline and then tested its tolerance to low precision. We also added a W16A8 setting specifically for the 16-layer set, while keeping W8A16 for the 114-layer base set as an extra mode. As a result, the 16-layer set actually improved over W8-only or A8-only, with W8A8 becoming the best quantized setting for that region, although it was still slightly below the no-quantization reference (Table III).

This finding appeared in the LIBERO_Goal suite. In contrast to the initial experiments, where no clear difference was found when A8, W8, or W8A8 was applied to the 16-layer set alone, this result indicates that the underlying 130-layer W8A16 baseline affected the model’s behaviour. Under this combined setting, W8A8 slightly rescued the lower performance of A8-only and W8-only. As for other suites, W8A16 on 130 layers results showed no significant effect on LIBERO_10 and LIBERO_Object, accuracy changed from 0.89 to 0.88, and 0.94 to 0.96, while LIBERO_Spatial and LIBERO_goal dropped accuracy from 0.94 to 0.0.84 and 0.95 to 0.88. And LIBERO_Spatial showed no significant changes upon applying A8 across different flow windows, contrast to LIBER_Goal.

However, we did not test whether the action head’s attention linear layers are correlated with the other action-head linear layers outside the attention module. These other layers are also used inside the timestep loop, but we only applied W8A16 to them because we were trying to follow the QuantVLA-style setting. Therefore, it is possible that these findings are due to the fact that action-head FFN linear layers and the selected attention linear layers interact with each other, or that W16A8 could produce unexpected results after all. This could help explain why W8A8 together rescued performance in some cases. However, since the benchmark experiments took a long time, it remains unclear whether this effect is caused by overfitting to LIBERO, by the limited number of failure cases, or by a real interaction that still needs further testing.

TABLE III

Table 3. Effect of Flow-Window Coverage on LIBERO-Goal under the W8A16 / 130-Layer Setting

| Target16 | Accuracy |
| --- | --- |
| W8A16 reference, no A8Baseline | 0.88 |
| W8A16 0-7 | 0.86 |
| W8A16 8–15 | 0.86 |
| W8A16 16–23 | 0.88 |
| W8A16 24–31 | 0.94 |
| W8A16 0-15 | 0.88 |
| W8A16 0-23 | 0.92 |
| W8A16 0-31 | 0.92 |
| W16A8 0-31 | 0.88 |

Meanwhile, this also confirms the initial experiment’s study of the temporal axis. Early timesteps showed more sensitivity and perturbing them alone had a negative effect. Again, full-timestep quantization improved accuracy. Quantizing all steps shows slightly better performance than quantizing only a small early window in Table III and Table IV, supporting the initial finding that timestep quantization may work better when applied consistently across the flow trajectory. It also suggests that later steps may be less sensitive than earlier steps.

## Stress Test

The top five most failed tasks were tested as a stress test, and the results showed that it is still too early to say that full-timestep quantization is indeed better than other settings. Early windows may be more sensitive, while full-step quantization can remain strong because the whole flow trajectory stays under one consistent precision regime. However, in stress or high-failure-boundary tasks, full-step quantization can fail more aggressively.

TABLE IV

Table 4. Effect of Flow-Window Coverage on LIBERO-Goal under the W8A16 / 130-Layer Setting

| Target16 | Accuracy |
| --- | --- |
| W8A16 reference, no A8Baseline | 0.88 |
| W8A16 0-7 | 0.86 |
| W8A16 8–15 | 0.86 |
| W8A16 16–23 | 0.88 |
| W8A16 24–31 | 0.94 |
| W8A16 0-15 | 0.88 |
| W8A16 0-23 | 0.92 |
| W8A16 0-31 | 0.92 |
| W16A8 0-31 | 0.88 |

The selected 16 attention linear layers are safe when tested alone, but their interaction with the 114-layer base set can reduce accuracy. LLM A8 is catastrophic, while LLM W8 is safe, although it is not perfect when combined with the other region. A8 on the selected 16 layers is not catastrophic and can sometimes improve the W8 setting, but again, only in the context of the combined-region experiment. Moreover, this effect is strongest in the Goal suite. It is unclear whether the type of task is a factor, or whether the model is simply overfitting to LIBERO too much. The timestep-window results show that early flow steps are generally more sensitive than later steps, which is the safer conclusion. Overall, these findings are closely related to the characteristics of VLA models.

# CONCLUSION

This study is not primarily about quantization for efficiency, because the studied layers do not occupy a large amount of memory or computational overhead. Instead, it asks a more specific question: how sensitive is the flow-matching head as a unique type of action head in VLA models?

The results show that sensitivity does not necessarily come from certain layers by themselves. Instead, in Evo-1, the more important issue is the combination of different quantized regions. If anything, the LLM side appears more sensitive in some settings, possibly because the model is already relatively small. This makes it important for multimodal models to consider degradation caused by combining quantization across different regions. The results indicate that quantization errors are not simply independent across the VLM side and the flow-action side. Small errors in the context representation and small errors inside the flow-matching action update can accumulate and become visible only in the combined setting.

For the selected 16-layer group, the results suggest that weight and activation quantization interact in a nontrivial way. For these layers, quantizing only weights or only activations can disturb the action head more than quantizing both sometimes under the same calibrated setting. However, the underlying reason for this behaviour is not explained by this study.

Overall, the results support three conclusions. First, the selected 16 action-head attention linear layers are quantizable and are not more sensitive than ordinary FFN-only targets when tested alone. However, the action head as a whole can still be sensitive when different quantized regions are combined, so it still requires controlled settings. Second, timestep-window sensitivity shows a trend where later steps are less sensitive and earlier steps are more sensitive, although this does not mean sensitivity changes linearly across the temporal axis. Third, the effect of quantization depends on both region interaction and task setting, so success or failure cannot be explained by location of the layers alone.

Future studies should not rely only on layer-wise accuracy or similarity metrics. They should also evaluate action drift, flow-trajectory drift, gripper mismatch, and other relevant robot-control metrics. Our study did not rigorously analyze the correlation between sensitivity, drift, and accuracy. However, in some task episodes, higher drift did not necessarily correspond to lower accuracy, and sometimes it even appeared to improve accuracy under certain settings. This report simply treats both action-space evidence and task-space evidence as our main criteria for evaluating quantization effects in Evo-1’s flow matching action head.

# REFERENCES

[1] Y. Xu, Y. Yang, Z. Fan, et al., “QVLA: Not All Channels Are Equal in Vision-Language-Action Model's Quantization”, 2026

[2] Z. Zheng, H. Cao, S. Tian, et al., “DyQ-VLA: Temporal-Dynamic-Aware Quantization for Embodied Vision-Language-Action Models”, 2026.

[3] J. Zhang, Y. Hsieh, Z. Wan, et al., “QuantVLA: Scale Calibrated Post-Training Quantization for Vision Language-Action Models”, 2026.

# APPENDIX

## Link to GitHub Repository

meowooomeowwooo-dev/VLA_Quantization